The Kria is notoriously tricky to debug in Vitis due to its hard-coded QSPI boot mode. This often causes inconsistent behavior, hanging at boot around 69% of the time, or memory write errors when Vitis and the SoM conflict over control. So, I must do the following after each board power down/up or pushing the reset button.

To bring up the “Task: XSDB Console” in Vitis 2025, press Debug. Then in that Console enter:

connect

targets -set -filter {name =~ "PSU"}

mwr 0xffca0010 0x0

mwr 0xff5e0200 0x0100

rst -system

This allows JTAG to take over the boot on rst -system. This is working for me on the IMU Golden vivado and vitis debug now.(Source: AMD).

What I was running into with default QSPI boot:

.text at valid address but memory write errors

A53 runaway / EDITR not ready

debug working one night and failing the next

Ubuntu/boot image interactions

golden design intermittently working

Status:

KR260 JTAG debug prep after every power cycle/reset:
disable/neutralize QSPI boot path so JTAG can take control cleanly.