IP/UDP Setup Instructions for the Kria and Jetson Orin Nano

Bring-Up Procedure

Physical Setup

- Ethernet cable type

- Direct connect or switch

- Power-up order

KR260 Network Setup

Commands:

sudo ip link set eth1 up

sudo ip addr add 192.168.10.2/24 dev eth1

Note: When looking at the front of the quad ethernet ports, the top right port is eth1.

Verify with: ip addr show

Jetson Network Setup

Commands:

sudo ip link set eth0 up

sudo ip addr add 192.168.10.1/24 dev eth0

## Verify with: ip addr show

## Connectivity Test

ping 192.168.10.1

Also capture:

- actual interface names

- actual IPs

- Ubuntu version

- Python version

- packet size

- update rate

- measured latency later

You now have:

- KR260 networking configured

- direct Ethernet communication

- UDP telemetry streaming

- Jetson packet reception

- a distributed embedded robotics pipeline

KR260 Ethernet Port Notes:
- Ubuntu 24.04 currently working on: <physical port location>
- Interface name: <eth0/end0/etc>
- Direct cable to Jetson Orin Nano
- Static IP configuration:
KR260 = 192.168.10.2
Jetson = 192.168.10.1

The AMD Kria KR260 Robotics Starter Kit features four RJ45 Ethernet ports on its carrier card, all of which are physically capable but serve different purposes based on whether they are connected to the Processing System (PS) or Programmable Logic (PL).

Functional Ethernet Ports

The four Ethernet ports are generally labeled J10A-D on the carrier card:

  - PS Ethernet (GEM1 & GEM0): Typically, the bottom-right port (J10C) is the default PS GEM1 port used for Ethernet connectivity/internet, which works out-of-the-box with pre-built Ubuntu images.

  - PL Ethernet (GEM3 & GEM2): Two Ethernet ports (J10A/J10B) are connected to the PL (FPGA) and require the appropriate firmware or bitstream to be loaded (typically TSN or EtherCAT applications) to function.