How to generate a dts file

Example dts can be found here: E:\ZenoRobotics\Projects\KR260_Jetson_Orin\kria_overlay_plus

A .dts file (Device Tree Source) describes the hardware layout Linux should expect on the AMD Zynq UltraScale+ MPSoC system.

For your KR260 project, the .dts is what tells Linux things like:

AXI DMA exists at 0xA0000000

UIO device exists

interrupts

clocks

memory mappings

custom IP blocks

Without the proper device tree, Linux may:

- not expose PL devices

- not create /dev/uioX

- not bind DMA drivers

- not configure interrupts/clocks correctly

Typical Flow

Vivado hardware design (.xsa)
↓
Device tree generated from hardware
↓
Linux boots using .dtb

Easiest Modern KR260 Method

From Vivado → Export XSA

File → Export Hardware

(include bitstream)

This creates:

design.xsa

Then Either:

Option 1 — PetaLinux (official flow)

PetaLinux auto-generates device tree files from the XSA.

Usually:

petalinux-config --get-hw-description=<xsa path>

Then build:

petalinux-build

Result:

.dtb

Option 2 — Manual DTS (good for Ubuntu/UIO experiments)

You can manually create a simple overlay or standalone DTS.

Example minimal AXI DMA node:

/dts-v1/;

/ {

axi_dma_0: dma@a0000000 {

compatible = "xlnx,axi-dma-1.00.a";

reg = <0x0 0xa0000000 0x0 0x10000>;

#dma-cells = <1>;

dma-channel@a0000030 {

compatible = "xlnx,axi-dma-s2mm-channel";

interrupts = <0 89 4>;

xlnx,datawidth = <32>;

};

};

};

For Simple Register Access (Best For You Right Now)

You do NOT even need full DMA Linux drivers yet.

Use UIO.

Example:

dma_uio@a0000000 {

compatible = "generic-uio";

reg = <0x0 0xa0000000 0x0 0x10000>;

};

That causes Linux to expose:

/dev/uio0

Then user-space can mmap the registers directly.

This is probably the fastest route for:

- DMA register testing

- AXI-lite validation

- PL debugging

Where DTS Files Live

Usually:

project-spec/meta-user/recipes-bsp/device-tree/files/system-user.dtsi

or:

arch/arm64/boot/dts/xilinx/

depending on flow.

Compiling DTS → DTB

dtc -I dts -O dtb -o system.dtb system.dts

For Ubuntu KR260

You can also use:

- device tree overlays

- fpga manager

- dynamic overlays

But for now:

simple UIO node

is probably the cleanest debug route.

My Recommendation For You

Right now I would NOT jump fully into:

- PetaLinux

- custom kernel drivers

- complex DTS generation

Instead:

Add a minimal UIO node for:

- DMA

- version register

- test register

Then confirm:

/dev/uio0

appears.

Once Linux can mmap/read your PL registers reliably:

- hardware is validated

- clocking validated

- AXI validated

- address map validated

Then return to the camera pipeline with confidence.