Q: What are the steps to reload the firmware. When I installed Ubuntu, I'm fairly certain I performed a firmware upgrade. If I need to go back to older fw that is fine.

A:

On Windows 10/11:

1. Connect Ethernet cable

Connect:

PC ↔ KR260 Ethernet port

directly.

2. Open Network Connections

Press:

Windows Key + R

Type:

ncpa.cpl

Press Enter.

This opens:

Network Connections

3. Find Ethernet adapter

Look for something like:

Ethernet

or:

Realtek PCIe GbE

Intel Ethernet

Right-click it:

Properties

4. Select IPv4

Click:

Internet Protocol Version 4 (TCP/IPv4)

Then:

Properties

5. Set manual IP

Select:

Use the following IP address

Enter:

IP address:     192.168.0.100

Subnet mask:    255.255.255.0

Default gateway: (leave blank)

You can leave DNS blank.

Click:

OK

then:

Close

6. Put KR260 into recovery mode

Usually:

- hold FWUEN button

- power cycle/reset board

(Depending on KR260 revision.)

7. Open browser

Go to:

http://192.168.0.111

If recovery mode started correctly, the firmware recovery webpage appears.

8. Upload firmware image

Use the webpage to flash:

- stable KR260 boot firmware

- preferably matching your Vivado/Vitis version

I would use:

2024.2-era firmware

for now.

===============================================

Q: Boot Image Status: Image A: Bootable , Image B: Non Bootable. The Boot Image Recovery Tool wants to know which image to recover Image A, B, or Wic

A:

Choose:

Image B

NOT:

- Image A

- Wic

Reason:

Image A = your currently working boot image

The recovery tool says:

Image A: Bootable

So preserve it.

Use:

Image B

as the recovery/test slot.

That gives you:

- a fallback image

- safer experimentation

- ability to switch later if needed

Only use:

Wic

if you are restoring a full Linux disk image/workflow, which you are not trying to do right now.

So the safe approach is:

Recover → Image B

with the older/stable KR260 boot firmware package.