Note: Both old & new boot images appear to work for bare-metal debugging (2023 & 2025).

- Initial board powerup directions:

- Vitis: Start Debug (press icon) to bring up TASK: XSDB Console

- Set default Vitis boot mode to JTAG instead of QSPI default.

- connect

- targets -set -filter {name =~ "PSU"}

- mwr 0xffca0010 0x0

- mwr 0xff5e0200 0x0100

- rst -system

- Vivado: Perform initial Program Device with both bitstream (.bit) + ila debug (.ltx) files.

- Restart debug in Vitis. (waiting for c code to be displayed and pc stopped at main() breakpoint.

- Vivado: Right click on Device (e.g., xc26_0).

- Set/arm trigger in ila display window (Vivado).

- Press arrow to run debug all the way through nonstop (unless you have set up other breakpoints).